Avaya PBX Integration with UltraVox

Complete Production Guide (With Examples & Required Components)

This document explains how to integrate Avaya IP Office with UltraVox AI voice agents using SIP.

Two methods are supported:

- IP Allowlisting (Direct SIP Trunk) – Recommended for on-prem PBX with static public IP.

- SIP Registration – Used when PBX is behind NAT or has dynamic IP.

1. What Is Needed Before Starting

1.1 From Avaya Side

You must have:

- Avaya IP Office system (on-prem or cloud)

- SIP trunk capability enabled

- Admin access to IP Office Manager

- Firewall access

- Internet connectivity

- Public IP (required for IP Allowlisting)

- SIP Registration Must Be Disabled

Network Requirements

- UDP 5060 open (SIP signalling)

- UDP 10000–20000 open (RTP media)

- SIP ALG disabled

- 1:1 NAT preferred (for static IP setups)

1.2 From UltraVox Side

You must have:

- Active UltraVox account

- API key (from dashboard)

- At least one AI agent created

- Access to UltraVox SIP configuration API

2. Method 1 – IP Allowlisting (Direct SIP Trunk)

Recommended for production if you have static public IP.

In this method:

- Avaya sends SIP INVITE directly to UltraVox.

- UltraVox accepts calls only from your allowlisted IP.

2.1 Call Flow Example:

Dubai DID: +971501234567

Avaya Public IP: 91.200.160.14

UltraVox Domain: sip.ultravox.ai

Agent ID: abc12345-6789

Call Path:

+971501234567 → Avaya → sip: agent_abc12345-6789@sip.ultravox.ai → AI Agent

2.2 Ultravox Configuration

Step 1 – Get SIP Domain

Request:  GET https://api.ultravox.ai/api/sip
                      Header: X-API-Key: YOUR_API_KEY

Example Response: { "domain": "sip.ultravox.ai"}

Step 2 – Allowlist Public IP

Request:  PATCH  https://api.ultravox.ai/api/sip
                      Header: X-API-Key: YOUR_API_KEY

Body Example: { "allowedCidrRanges": [“91.200.160.14/32"],
                                   "allowAllAgents": false ,

"allowedAgents": [{ "agentic": "<agent_id>" }}

- Use /32 format for a single IP. Do not use 0.0.0.0/0

- This allows only your Avaya public IP to send SIP calls to Ultravox. Without this step, UltraVox will reject SIP calls from your PBX.

2.3 Avaya Configuration

Step 1 – Create SIP Line

Navigate:

Line → SIP Line → New

Example Settings:

- Line Name: UltraVox

- ITSP Domain: sip.ultravox.ai (SIP Domain from Ultravox)

- Transport: UDP

- Port: 5060

- Registration: Disabled

- Authentication: None

Step 2 – Configure Incoming Call Route (Avaya IP Office -- For Live DID Calls)

- In Avaya IP Office, the Incoming Call Route defines where calls to your public DID number will be forwarded.

Navigate: Incoming Call Route → New

Configure the Following: Incoming Number (DID): +971501234567

- This tells Avaya to apply this rule when that Dubai number is called.

Line Group ID: Select the SIP trunk that receives calls from your telecom provider.

Example:

Set destination to: sip: agent_<agent_uuid>@sip.ultravox.ai

- Assign Dubai DID: +971501234567

- This means every call to that number will go to the UltraVox agent.

What This Means

When someone calls +971501234567:

- The telecom provider delivers the call to Avaya.

- Avaya matches the DID in Incoming Call Route.

- Avaya sends a SIP INVITE to UltraVox.

- The UltraVox AI agent answers the call.

This setup is used for live production calls.

Step 3 – Internal Testing Without Calling the Real DID (Using Softphone)

Testing: test the integration without calling the real Dubai number, you can create an internal test route.

- This allows you to dial a test number (for example, 3333) from your Avaya softphone.

Option A – Create a Short Code (Recommended for Testing)

Short Code → New

Example Configuration:

Code:3333

Feature: Dial

Telephone Number: sip:agent_abc12345-6789@sip.ultravox.ai

Line Group ID:
Select the UltraVox SIP trunk line.

What Happens When You Dial 3333

- You dial 3333 from your Avaya softphone.

- Avaya matches the short code.

- Avaya sends SIP INVITE to sip.ultravox.ai.

- UltraVox AI answers the call.

- RTP audio flows between Avaya and UltraVox.

This allows internal testing without using the external DID.

2.4 Codec Configuration (Production Recommendation)

Use only: G.711 A-law (PCMA)

Additional Configuration

• Disable all other codecs (G.722, Opus, G.729, etc.)
• Ensure transcoding is not forced internally
• Set packetization time to 20ms
• Confirm internal endpoints also support G.711

2.5 Final Required Checklist (IP Allowlisting)

Final Go-Live Checklist (IP Allowlisting – Avaya → UltraVox)

1️⃣ Network & IP Configuration

  - Configure and confirm static public IP on Avaya.

  - Ensure outbound SIP traffic on port 5060 is allowed.

  - Open outbound RTP port range (e.g., 10000–20000) on firewall.

  - Disable SIP ALG on firewall/router.

  - Disable any SIP inspection or header rewriting features.

  - Ensure DNS is configured on Avaya and resolves sip.ultravox.ai.

  - Ensure sufficient internet upload bandwidth is provisioned for expected concurrent calls.

2️⃣ Avaya SIP Configuration

  - Configure SIP trunk to UltraVox using provided IP/domain.

  - Ensure SIP transport and port 5060 are correctly configured.

  - Enable SIP OPTIONS keepalive (recommended 30 seconds).

  - Enable SIP session timers (~1800 seconds, refresh via re-INVITE).

  - Configure DTMF mode as RFC2833. (NOT REQUIRED NOW )

  - Configure Caller ID / P-Asserted-Identity in E.164 format.

  - Enable REFER support (if call transfer is required). (NOT REQUIRED NOW)

  - Configure RTP port range (e.g., 10000–20000) in Avaya.

  - Ensure Avaya advertises public IP address in SDP (NAT/Public IP mapping configured properly).